isSPA Manual

version: 1.2

Authors: Yuanhao Cheng

Date: November 30th, 2025

cryoSPARC

- Do “Import Movies, Patch Motion Correction, Patch CTF Estimation, Manually Curate Exposures” in cryoSPARC

- Activate pyem environment, and convert .cs file to .star file:

csparc2star.py CS-test-data/J3/exposures_ctf_estimated.cs CS-test-data /J3/J3_passthrough_exposures.cs CS-test-data/J4/J4_passthrough_exposures_accepted.cs micrograph_ctf.star

- Add the corresponding directory to micrograph_ctf.star file: vim micrograph_ctf.star :1,$s/J3/CS-test-data\/J3/g

- Start GUI: isSPA.py &

- Click “Import parameters from RELION” and select micrograph_ctf.star.

- Select an appropriate 3D Template by clicking “select”.

- (Optional) If you have the corresponding FSC file (STAR format), select it.

- Choose Protein Mass range, and input Diameter.

- Confirm the No. of images to be processed.

- Modify the output file name.

- (Optional) Click Save Config and save the parameters you have inputted in the GUI, for your convenience.

- Click Start Processing, and calculation will start automatically.

- (Optional) Click Display Particles and check the picking results. Select the file that ends with “merge_31.star”.

- Use RELION to do Particle Extraction and 3D classification

  - Reference: Initial low-pass filter (A): 30-60

  - Sampling: Perform image alignment? No!!!

RELION

- Start GUI: isSPA.py &

- Click “Start RELION” and use RELION to do “Import, Motion Correction, CTF estimation”.

- (Optional) Select micrographs with mrc_picker.py.

- In the isSPA GUI, click “Import parameters from RELION” and select micrograph_ctf.star in the CtfFind directory.

- The following steps are the same as those in cryoSPARC step 6.